- MaUWB_AOA Gen2 Tag Data Sheet

version: 1.0

by: Leobo

目录

MaUWB_AOA Gen2 Tag Data Sheet	1

1. Product Introduction	1

1.1 Product Introduction	1

1.2 Product Features	2

1.3 Application objects	3

2. Module Hardware Specifications	4

2.1 Module Hardware Pin Definitions	4

2.2 Module hardware packaging dimensions	5

3. Communication Protocol Description	6

3.1 Base Station Positioning Frame	6

3.2 Base station positioning frame RSSI	8

3.3 Tag Configuration Frame	10

3.4 OTA upgrade frame	10

- Product Introduction

  - Product Introduction

- Makerfabs Electronics' MaUWB_AOA Gen2 Tag is a patch-type tag module specifically designed for the MaUWB-AOA positioning system and is used in conjunction with MaUWB_AOA Gen2 Anchor. The module continuously emits UWB signals, and the base station receives them to calculate the relative distance and angle of the wearer, enabling automatic following of equipment such as luggage carts, AGVs, and service robots. The bare board module has a shape suitable for customers to embed into their own products for secondary customization development. The entire system has a measurement error of less than 10 cm, an angle error of less than 5°, a maximum visible distance of 50 meters, and is not affected by light interference, outperforming visual and ultrasonic following solutions.

| Follow tag: MaUWB_AOA Gen2 Tag module |  |
| --- | --- |
| Product Images |  |
| External dimensions | 13mm*23mm*2mm |
| Product Weight | 1.5g |
| Power supply method | 3.3V/200MA |
| Operating Current | 12MA |
| Communication Methods | UART TTL |
| Operating frequency | CH9 band 8987.2MHz |
| Distance Measurement Error | <10CM |
| Angle Error | <5° |
| Operating Temperature | -20℃~+65℃ |
| Use Cases | Follow the scene application (combined with base station usage) |
| Features | Labeling is optional; the frequency band complies with MIIT regulations; the module is ultra-compact at 1.3 × 2.3 cm; simply plug it in to use. |

  - Product Features

- Small, thin and easy to integrate:The device is compact in size and can be easily integrated into various wearable and remote control devices. This makes it convenient for the secondary development of the entire machine.

- Strong environmental anti-interference capability:Not affected by strong light, indoor lighting, or mild non-metallic obstructions, compared to visual and ultrasonic tracking methods, it has greater stability in outdoor and complex indoor environments.

- Multiple devices without interference:Supports custom network IDs. Multiple sets of slave devices can work simultaneously without interfering with each other.

- High-precision AOA positioning output:By combining with the supporting base stations, TOF ranging + AOA angle measurement is achieved. The ranging error is less than 10cm, the angle measurement error is less than 5°, the visual communication range is up to 50 meters, and the positioning accuracy is high.

- Full-directional antenna, with wide signal coverage range:During daily use, there is no need to deliberately adjust the placement or wearing angle. The signal can be stably transmitted even in the normal usage posture.

  - Application objects

- Following applications of balance scooters and following robots

- Application of service robots

- Specialized robots and unmanned aerial vehicle applications

- Module Hardware Specifications

  - Module Hardware Pin Definitions

- Module Pinout Description 1

- Module Pinout Description 2

| Pin | Function |
| --- | --- |
| 1 | GND |
| 3 | Download CLK |
| 4 | Download DIO |
| 5 | Green status indicator light; power supply is flashing |
| 6 | Red power indicator light; power is on and the light remains lit |
| 2、7、8、9、10、14、15 | NC must be suspended |
| 11 | UART_Tx Pin speed 115200 |
| 12 | UART_Rx Pin speed 115200 |
| 13 | Reset pin, with low level as the effective signal |
| 16 | Positive power supply terminal is powered by 3.3V |

  - Module hardware packaging dimensions

Module size specifications

- Communication Protocol Description

  - Base Station Positioning Frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd_saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame types (1: Base station positioning frame 2: Base station configuration frame 3: Tag configuration frame 4: OTA upgrade frame 5: Whole machine configuration frame 0x64: Base station positioning frame RSSI) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| tag_time | uint32_t | 4 | 21 | Tag communication time |
| anc_addr16 | Uint16_t | 2 | 25 | The short address of the base station |
| tag_addr16 | uint16_t | 2 | 27 | The short address of the tag |
| tag_sn | uint8_t | 1 | 29 | Tag communication sn |
| tag_mask | uint8_t | 1 | 30 | Tag communication is effective. |
| tag_a0_filter_angle | int16_t | 2 | 31 | Base station 0 filtering angle (°) |
| tag_a0_filter_range | uint16_t | 2 | 33 | Base station 0 filtering distance (cm) |
| tag_a1_filter_angle | int16_t | 2 | 35 | Base station 1 filtering angle (°) |
| tag_a1_filter_range | uint16_t | 2 | 37 | Base station 1 filtering distance (cm) |
| tag_a2_filter_angle | int16_t | 2 | 39 | Base station 2 filtering angle (°) |
| tag_a2_filter_range | uint16_t | 2 | 41 | Base station 2 filtering distance (cm) |
| tag_a3_filter_angle | int16_t | 2 | 43 | Base station 3 filtering angle (°) |
| tag_a3_filter_range | uint16_t | 2 | 45 | Base station 3 filtering distance (cm) |
| tag_detail_para | uint32_t | 4 | 47 | Tag data (tag type / battery power / charging status / remote control status) [Note 1: bit structure] |
| tag_acc_x | uint16_t | 2 | 51 | Tag acceleration along the x-axis |
| tag_acc_y | uint16_t | 2 | 53 | Tagged acceleration along the y-axis |
| tag_acc_z | uint16_t | 2 | 55 | Tagged acceleration along the z-axis |
| tag_gcc_x | uint16_t | 2 | 57 | Taggy Gyroscope X-axis |
| tag_gcc_y | uint16_t | 2 | 59 | Taggy Gyroscope Y-axis |
| tag_gcc_z | uint16_t | 2 | 61 | Taggy Gyroscope Z-axis |
| self_raw_angle | int16_t | 2 | 63 | The original measurement angle of the object (unfiltered) |
| self_raw_range | uint16_t | 2 | 65 | The original measurement distance of the object (unfiltered) |
| self_raw_degree | int32_t | 4 | 67 | Original angle of the tag (used during calibration of the base station) |
| angle_360 | uint16_t | 2 | 71 | 360° angle (used in conjunction with the entire machine) |
| angle_dir | uint8_t | 1 | 73 | 360° angle approximate direction (for use with the entire machine) |
| reserve | uint8_t* | 5 | 74 | Reserve 5 bytes |
| check | uint8_t | 1 | 79 | Check bit (field: checksum of cmd_saddr-reserve)[Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | 80 | Fixed trailer byte: 0x23 |

  - Base station positioning frame RSSI

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd_saddr saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame type (1: Base station positioning frame 2: Base station configuration frame 3: Tag configuration frame 4: OTA upgrade frame 5: Whole machine configuration frame 0x64: Base station positioning frame RSSI) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| tag_time | uint32_t | 4 | 21 | Tag communication time |
| anc_addr16 | Uint16_t | 2 | 25 | The short address of the base station |
| tag_addr16 | uint16_t | 2 | 27 | The short address of the tag |
| tag_sn | uint8_t | 1 | 29 | Tag communication sn |
| tag_mask | uint8_t | 1 | 30 | Tag communication is effective |
| tag_a0_filter_angle | int16_t | 2 | 31 | Base station 0 filtering angle (°) |
| tag_a0_filter_range | uint16_t | 2 | 33 | Base station 0 filtering distance (cm) |
| tag_a0_rssl | int16_t | 2 | 35 | Base station 0 signal strength |
| tag_a1_filter_angle | int16_t | 2 | 37 | Base station 1 filtering angle (°) |
| tag_a1_filter_range | uint16_t | 2 | 39 | Base station 1 filtering distance (cm) |
| tag_a1_rssl | int16_t | 2 | 41 | Base station 1 signal strength |
| tag_a2_filter_angle | int16_t | 2 | 43 | Base station 2 filtering angle (°) |
| tag_a2_filter_range | uint16_t | 2 | 45 | Base station 2 filtering distance (cm) |
| tag_a2_rssl | int16_t | 2 | 47 | Base station 2 signal strength |
| tag_a3_filter_angle | int16_t | 2 | 49 | Base station 3 filtering angle (°) |
| tag_a3_filter_range | uint16_t | 2 | 51 | Base station 3 filtering distance (cm) |
| tag_a3_rssl | int16_t | 2 | 53 | Base station 3 signal strength |
| tag_detail_para | uint32_t | 4 | 55 | Tag data (tag type / battery power / charging status / remote control status) [Note 1: bit structure] |
| tag_acc_x | uint16_t | 2 | 59 | Tag acceleration along the x-axis |
| tag_acc_y | uint16_t | 2 | 61 | Tagged acceleration along the y-axis |
| tag_acc_z | uint16_t | 2 | 63 | Tagged acceleration along the z-axis |
| tag_gcc_x | uint16_t | 2 | 65 | Taggy Gyroscope X-axis |
| tag_gcc_y | uint16_t | 2 | 67 | Taggy Gyroscope Y-axis |
| tag_gcc_z | uint16_t | 2 | 69 | Taggy Gyroscope Z-axis |
| self_raw_angle | int16_t | 2 | 71 | The original measurement angle of the object (unfiltered) |
| self_raw_range | uint16_t | 2 | 73 | The original measurement distance of the object (unfiltered) |
| self_raw_degree | int32_t | 4 | 75 | Original angle of the tag (used during calibration of the base station) |
| angle_360 | uint16_t | 2 | 79 | 360° angle (used in conjunction with the entire machine) |
| angle_dir | uint8_t | 1 | 81 | 360° angle approximate direction (for use with the entire machine) |
| reserve | uint8_t* | 5 | 82 | Reserve 5 bytes |
| check | uint8_t | 1 | 87 | Check bit (field: checksum of cmd_saddr-reserve)[Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | 88 | Fixed trailer byte: 0x23 |

  - Tag Configuration Frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd_saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame type (1: Base station positioning frame 2: Base station configuration frame 3: Tag configuration frame 4: OTA upgrade frame 5: Whole machine configuration frame 0x64: Base station positioning frame RSSI) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| config_cmd | uint8_t* | * | * | Configuration command set [Note 4: Label configuration command set] |
| check | uint8_t | 1 | * | Checksum (Field: checksum of cmd_ saddr-config_cmd) [Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | * | Fixed trailer byte: 0x23 |

  - OTA upgrade frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd and saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame type (1: Base station positioning frame 2: Base station configuration frame 3: Tag configuration frame 4: OTA upgrade frame 5: Whole machine configuration frame 0x64: Base station positioning frame RSSI) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| OTA_cmd | uint8_t* | * | * | OTA commands (developers can ignore this) |
| check | uint8_t | 1 | * | Checksum (Field: checksum of cmd_ saddr-OTA_cmd) [Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | * | Fixed trailer byte: 0x23 |

[Note 1: Bit structure]

【Alarm Status: No/(Remote Control Dedicated) Mode: No/】

0x041A0002 (Hexadecimal) = 0x02001A04 (High-low bit conversion) = 00000010000000000001101000000100 (Binary)

【Alarm Status: Yes/(Remote Control Dedicated) Mode: No/】

0x061A0002 (in hexadecimal) = 0x02001A06 (High-low bit conversion) = 00000010000000000001101000000110 (in binary)

【Alarm status: No / Remote control dedicated mode: Yes / 】

0x241A1002 (in hexadecimal) = 0x02101A24 (High-low bit conversion) = 00000010000100000001101000000110 (in binary)

typedefstruct

{

uint32_tis_lowbattery:1; 			//Is there a low battery alarm?

uint32_tis_alarm:1; 				//Is there an alarm?

uint32_tis_chrg:1;	 				//Is charging required?

uint32_tis_tdby:1; 					//Is it fully charged?

uint32_tbattery_val:10; 				//Battery voltage 350 = 3.50V

uint32_tis_offset_range_zero_bit:1; 	//Distance correction position

uint32_tis_offset_pdoa_zero_bit:1; 	//Angle correction position

uint32_tturn_up:1; 					//(For remote control use) Forward

uint32_tturn_down:1; 				//(For remote control use) Backward

uint32_tturn_left:1; 				//(For remote control use) Left

uint32_tturn_right:1; 				//(For remote control use) Right

uint32_tmode:3; 					//(For remote control use) Mode

uint32_trecal:1; 					//(For remote control use) Recall

uint32_tlock:1; 					//(For remote control use) Locked

uint32_tdev_type:3; 	    //Device type (0: Learning board 1: Bracelet 2: Remote control)

uint32_treserve:4; 					//Reserved

}tag_detail_para_t;

[Note 2: Verification calculation program]

Static uint8_t get_Xor_CRC(uint8_t *bytes, int offset, int len)

{

uint8_t xor_crc = 0;

int i;

for (i = 0;i < len; i++) {

xor_crc ^= bytes[offset + i];

}

return xor_crc;

}

[Note 3: Tag Configuration Command Set]

| Command | Meaning | Examples |
| --- | --- | --- |
| reset | Reset | Reset |
| rtoken | Restore to factory settings | Restore to factory settings |
| save | Save | Save settings |
| saver | Save & Reset | Save & Reset |
| getver | Get the version |  |
| getid | Get the tag ID |  |
| sethw(x1)(x2)(x3)(x4) | Set hardware parameters | sethw 3070 0 120 31 x1: Antenna delay (default: 3070) x2: External power amplifier (default: off) x3: Internal power (default: 120) |
| gethw | Obtain hardware parameters | gethw |
| setsen(x1)(x2) | Set tag parameters | setsen 00000000 0000 x1: tag_detail_para_t structure x2: joy_stick_t structure [optional] |
| setsrch(x1)(x2) | Set forced binding parameters | setsrch 1 30000 x1: Whether to force tag to join the network x2: Force join time setting (ms) |
| setslp(x1) | Set sleep time (in ms) (-1: Trigger only by external event) | setslp -1 x1: -1 (only external wake-up) |

[Note 4: Example of Base Station Positioning Frame]

2A 4C 00 DD DD DD DD DD DD DD DD EE EE EE EE EE EE EE EE 01 05 93 23 1C 00 00 00 D8 64 87 01 F9 FF 60 00 00 00 00 00 00 00 00 00 00 00 00 00 FC 17 00 02 00 00 00 00 00 00 00 00 00 00 00    00 F9 FF 60 00 00 00 00 00 45 00 03 00 00 00 00 00 3D 23

| Original data | Frame meaning | Data conversion | Index subscript |
| --- | --- | --- | --- |
| 2A | Frame header |  | 0 |
| 4C 00 | Data frame length |  | 1 |
| DD DD DD DD DD DD DD DD | Self-long address (reserved 8 bytes) |  | 3 |
| EE EE EE EE EE EE EE EE | Recipient's full address (8 bytes reserved) |  | 11 |
| 01 | Command frame types |  | 19 |
| 05 | Command frame direction |  | 20 |
| 93 23 1C 00 | Tag communication time |  | 21 |
| 00 00 | The short address of the base station |  | 25 |
| D8 64 | The short address of the tag |  | 27 |
| 87 | Tag communication sn |  | 29 |
| 01 | Tag communication is effective. |  | 30 |
| F9 FF | Base station 0 filtering angle (°) | 0xFFF9 (in hexadecimal) = -6° | 31 |
| 60 00 | Base station 0 filtering distance (cm) | 0x0060 (in hexadecimal) = 96 cm | 33 |
| 00 00 | Base Station 1 filtering Angle (°) | Reference base station 0 angle conversion | 35 |
| 00 00 | Base station 1 filtering distance (cm) | Reference base station 0 distance conversion | 37 |
| 00 00 | Base Station 2 filtering Angle (°) | Reference base station 0 angle conversion | 39 |
| 00 00 | Base station 2 filtering distance (cm) | Reference base station 0 distance conversion | 41 |
| 00 00 | Base Station 3 filtering Angle (°) | Reference base station 0 angle conversion | 43 |
| 00 00 | Base station 3 filtering distance (cm) | Reference base station 0 distance conversion | 45 |
| FC 17 00 02 | Tag data (Note 1: Bit structure) |  | 47 |
| 00 00 00 00 00 00 | Tagged acceleration along the x-axis/y-axis/z-axis |  | 51 |
| 00 00 00 00 00 00 | Tag gyroscope x-axis/y-axis/z-axis |  | 57 |
| F9 FF | The original measurement angle of the object (unfiltered) | Reference base station 0 angle conversion | 63 |
| 60 00 | The original measurement distance of the object (unfiltered) | Reference base station 0 distance conversion | 65 |
| 00 00 00 00 | Original angle of the label (used during calibration of the base station) |  | 67 |
| 45 00 | 360° angle (used in conjunction with the entire machine) | Reference base station 0 angle conversion | 71 |
| 03 | 360° angle approximate direction (for use with the entire machine) | Reference base station 0 angle conversion | 73 |
| 00 00 00 00 00 | Reserve 5 bytes |  | 74 |
| 3D | Check bit |  | 79 |
| 23 | Frame trailer |  | 80 |