MaUWB AOA Anchor Data Sheet

version: 1.0

by: Leobo

目录

MaUWB AOA Anchor Data Sheet	1

1. Product Introduction	1

1.1 Product Introduction	1

1.2 Product Features	2

1.3 Application objects	3

2. Module Hardware Specifications	3

2.1 Module Hardware Pin Definitions	4

2.2 Module hardware packaging dimensions	4

3. Communication Protocol Description	5

3.1 Base Station Positioning Frame	5

3.2 Base station positioning frame RSSI	6

3.3 Base station configuration frame	8

3.4 OTA upgrade frame	9

- Product Introduction

  - Product Introduction

- Makerfabs Electronics' MaUWB-AOA-U is an ultra-small patch-type AOA base station module specially developed for the MaUWB-AOA positioning system, working cooperatively with MaUWB-AOA-T tags. Equipped with a built-in mini antenna array and metal shielding cover for anti-interference performance, the module receives UWB signals transmitted by wearable tags, calculates the real-time relative distance and incident angle of the target, and outputs filtered positioning data via TTL serial port to vehicle main controllers to realize automatic following for luggage trolleys, AGVs, service robots, balance cars and unmanned aerial vehicles. The bare-board ultra-compact form factor allows customers to embed it tightly inside compact devices for secondary integration and customized development.

| Follow-up base station: MaUWB-AOA-U module |  |
| --- | --- |
| Product Images |  |
| External Dimensions | 39mm*30mm*5mm |
| Product Weight | 4 g |
| Positioning Angle | 150° in front of the base station |
| Coverage radius | 50 meters - Wide open viewable area |
| Communication Methods | UART-TTL |
| Power Supply Method | 5V Power Supply |
| Installation Methods | M2 Fixed-Hole Installation |
| Frequency Range | CH9 (7987.2 MHz) (Complies with MIIT requirements) |
| Distance Measurement Error | <10CM |
| Angle Error | <5° |
| Operating Temperature | -20℃~+65℃ |
| Use Cases | Follow scene application (use in conjunction with tags) |
| Features | The frequency band complies with MIIT regulations; ultra-compact size of 3.9 × 3 cm; compatible with the URAT protocol; and features a shielded housing to resist interference. |

  - Product Features

- High-Precision Distance and Angle Detection: This module uses a miniature antenna array and ultra-wideband (UWB) radio technology to accurately measure the distance and angle between wearable tags and anchor points. It provides reliable positioning data for automatic tracking and control.

- Wide coverage and large detection field of view: It features a 150°front detection angle with an open-sight coverage radius exceeding 50 meters, offering ample moving space for various following devices and effectively avoiding tag signal loss.

- High resistance to interference; unaffected by ambient light: Equipped with a shielding cover, it is unaffected by changes in lighting and provides stable,reliable data in complex real-world environments.

- Simultaneous multi-target tracking to prevent tracking confusion:Avoid interference among multiple sets through the network ID. Support the simultaneous operation of multiple tracking devices.

- Easy to integrate with standard serial communication: Users can connect a TTL-to-USB adapter to view positioning data in real time using PC software, significantly reducing the complexity of secondary development.

  - Application objects

- Following applications of balance scooters and following robots

- Application of service robots

- Specialized robots and unmanned aerial vehicle applications

- Module Hardware Specifications

  - Module Hardware Pin Definitions

| Lable | Function |
| --- | --- |
| TXD(1) | UART_Tx Pin speed 115200 |
| RXD(3) | UART_Rx Pin speed 115200 |
| GND(4) | GND |
| VCC(5) | Positive power supply terminal is powered by 5V |

  - Module hardware packaging dimensions

Module size specifications

- Communication Protocol Description

  - Base Station Positioning Frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd_saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame types (1: Base station positioning frame 2: Base station configuration frame 3: Tag configuration frame 4: OTA upgrade frame 5: Whole machine configuration frame 0x64: Base station positioning frame RSSI) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| tag_time | uint32_t | 4 | 21 | Tag communication time |
| anc_addr16 | Uint16_t | 2 | 25 | The short address of the base station |
| tag_addr16 | uint16_t | 2 | 27 | The short address of the tag |
| tag_sn | uint8_t | 1 | 29 | Tag communication sn |
| tag_mask | uint8_t | 1 | 30 | Tag communication is effective. |
| tag_a0_filter_angle | int16_t | 2 | 31 | Base station 0 filtering angle (°) |
| tag_a0_filter_range | uint16_t | 2 | 33 | Base station 0 filtering distance (cm) |
| tag_a1_filter_angle | int16_t | 2 | 35 | Base station 1 filtering angle (°) |
| tag_a1_filter_range | uint16_t | 2 | 37 | Base station 1 filtering distance (cm) |
| tag_a2_filter_angle | int16_t | 2 | 39 | Base station 2 filtering angle (°) |
| tag_a2_filter_range | uint16_t | 2 | 41 | Base station 2 filtering distance (cm) |
| tag_a3_filter_angle | int16_t | 2 | 43 | Base station 3 filtering angle (°) |
| tag_a3_filter_range | uint16_t | 2 | 45 | Base station 3 filtering distance (cm) |
| tag_detail_para | uint32_t | 4 | 47 | Tag data (tag type / battery power / charging status / remote control status) [Note 1: bit structure] |
| tag_acc_x | uint16_t | 2 | 51 | Tag acceleration along the x-axis |
| tag_acc_y | uint16_t | 2 | 53 | Tagged acceleration along the y-axis |
| tag_acc_z | uint16_t | 2 | 55 | Tagged acceleration along the z-axis |
| tag_gcc_x | uint16_t | 2 | 57 | Taggy Gyroscope X-axis |
| tag_gcc_y | uint16_t | 2 | 59 | Taggy Gyroscope Y-axis |
| tag_gcc_z | uint16_t | 2 | 61 | Taggy Gyroscope Z-axis |
| self_raw_angle | int16_t | 2 | 63 | The original measurement angle of the object (unfiltered) |
| self_raw_range | uint16_t | 2 | 65 | The original measurement distance of the object (unfiltered) |
| self_raw_degree | int32_t | 4 | 67 | Original angle of the tag (used during calibration of the base station) |
| angle_360 | uint16_t | 2 | 71 | 360° angle (used in conjunction with the entire machine) |
| angle_dir | uint8_t | 1 | 73 | 360° angle approximate direction (for use with the entire machine) |
| reserve | uint8_t* | 5 | 74 | Reserve 5 bytes |
| check | uint8_t | 1 | 79 | Check bit (field: checksum of cmd_saddr-reserve)[Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | 80 | Fixed trailer byte: 0x23 |

  - Base station positioning frame RSSI

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd_saddr saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame type (1: Base station positioning frame 2: Base station configuration frame 3: Tag configuration frame 4: OTA upgrade frame 5: Whole machine configuration frame 0x64: Base station positioning frame RSSI) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| tag_time | uint32_t | 4 | 21 | Tag communication time |
| anc_addr16 | Uint16_t | 2 | 25 | The short address of the base station |
| tag_addr16 | uint16_t | 2 | 27 | The short address of the tag |
| tag_sn | uint8_t | 1 | 29 | Tag communication sn |
| tag_mask | uint8_t | 1 | 30 | Tag communication is effective |
| tag_a0_filter_angle | int16_t | 2 | 31 | Base station 0 filtering angle (°) |
| tag_a0_filter_range | uint16_t | 2 | 33 | Base station 0 filtering distance (cm) |
| tag_a0_rssl | int16_t | 2 | 35 | Base station 0 signal strength |
| tag_a1_filter_angle | int16_t | 2 | 37 | Base station 1 filtering angle (°) |
| tag_a1_filter_range | uint16_t | 2 | 39 | Base station 1 filtering distance (cm) |
| tag_a1_rssl | int16_t | 2 | 41 | Base station 1 signal strength |
| tag_a2_filter_angle | int16_t | 2 | 43 | Base station 2 filtering angle (°) |
| tag_a2_filter_range | uint16_t | 2 | 45 | Base station 2 filtering distance (cm) |
| tag_a2_rssl | int16_t | 2 | 47 | Base station 2 signal strength |
| tag_a3_filter_angle | int16_t | 2 | 49 | Base station 3 filtering angle (°) |
| tag_a3_filter_range | uint16_t | 2 | 51 | Base station 3 filtering distance (cm) |
| tag_a3_rssl | int16_t | 2 | 53 | Base station 3 signal strength |
| tag_detail_para | uint32_t | 4 | 55 | Tag data (tag type / battery power / charging status / remote control status) [Note 1: bit structure] |
| tag_acc_x | uint16_t | 2 | 59 | Tag acceleration along the x-axis |
| tag_acc_y | uint16_t | 2 | 61 | Tagged acceleration along the y-axis |
| tag_acc_z | uint16_t | 2 | 63 | Tagged acceleration along the z-axis |
| tag_gcc_x | uint16_t | 2 | 65 | Taggy Gyroscope X-axis |
| tag_gcc_y | uint16_t | 2 | 67 | Taggy Gyroscope Y-axis |
| tag_gcc_z | uint16_t | 2 | 69 | Taggy Gyroscope Z-axis |
| self_raw_angle | int16_t | 2 | 71 | The original measurement angle of the object (unfiltered) |
| self_raw_range | uint16_t | 2 | 73 | The original measurement distance of the object (unfiltered) |
| self_raw_degree | int32_t | 4 | 75 | Original angle of the tag (used during calibration of the base station) |
| angle_360 | uint16_t | 2 | 79 | 360° angle (used in conjunction with the entire machine) |
| angle_dir | uint8_t | 1 | 81 | 360° angle approximate direction (for use with the entire machine) |
| reserve | uint8_t* | 5 | 82 | Reserve 5 bytes |
| check | uint8_t | 1 | 87 | Check bit (field: checksum of cmd_saddr-reserve)[Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | 88 | Fixed trailer byte: 0x23 |

  - Base station configuration frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd_saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame type (1: Base station positioning frame 2: Base station configuration frame 3: Tag configuration frame 4: OTA upgrade frame 5: Whole machine configuration frame 0x64: Base station positioning frame RSSI) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| config_cmd | uint8_t* | * | * | Configuration command set [Note 3: Base station configuration command set] |
| check | uint8_t | 1 | * | Check bit (Field: cmd_saddr - checksum of config_cmd) [Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | * | Fixed trailer byte: 0x23 |

  - OTA upgrade frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd and saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame type (1: Base station positioning frame 2: Base station configuration frame 3: Tag configuration frame 4: OTA upgrade frame 5: Whole machine configuration frame 0x64: Base station positioning frame RSSI) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| OTA_cmd | uint8_t* | * | * | OTA commands (developers can ignore this) |
| check | uint8_t | 1 | * | Checksum (Field: checksum of cmd_ saddr-OTA_cmd) [Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | * | Fixed trailer byte: 0x23 |

[Note 1: Bit structure]

【Alarm Status: No/(Remote Control Dedicated) Mode: No/】

0x041A0002 (Hexadecimal) = 0x02001A04 (High-low bit conversion) = 00000010000000000001101000000100 (Binary)

【Alarm Status: Yes/(Remote Control Dedicated) Mode: No/】

0x061A0002 (in hexadecimal) = 0x02001A06 (High-low bit conversion) = 00000010000000000001101000000110 (in binary)

【Alarm status: No / Remote control dedicated mode: Yes / 】

0x241A1002 (in hexadecimal) = 0x02101A24 (High-low bit conversion) = 00000010000100000001101000000110 (in binary)

typedefstruct

{

uint32_tis_lowbattery:1; 			//Is there a low battery alarm?

uint32_tis_alarm:1; 				//Is there an alarm?

uint32_tis_chrg:1;	 				//Is charging required?

uint32_tis_tdby:1; 					//Is it fully charged?

uint32_tbattery_val:10; 				//Battery voltage 350 = 3.50V

uint32_tis_offset_range_zero_bit:1; 	//Distance correction position

uint32_tis_offset_pdoa_zero_bit:1; 	//Angle correction position

uint32_tturn_up:1; 					//(For remote control use) Forward

uint32_tturn_down:1; 				//(For remote control use) Backward

uint32_tturn_left:1; 				//(For remote control use) Left

uint32_tturn_right:1; 				//(For remote control use) Right

uint32_tmode:3; 					//(For remote control use) Mode

uint32_trecal:1; 					//(For remote control use) Recall

uint32_tlock:1; 					//(For remote control use) Locked

uint32_tdev_type:3; 	    //Device type (0: Learning board 1: Bracelet 2: Remote control)

uint32_treserve:4; 					//Reserved

}tag_detail_para_t;

[Note 2: Verification calculation program]

Static uint8_t get_Xor_CRC(uint8_t *bytes, int offset, int len)

{

uint8_t xor_crc = 0;

int i;

for (i = 0;i < len; i++) {

xor_crc ^= bytes[offset + i];

}

return xor_crc;

}

[Note 3: Base Station Configuration Command Set]

| Command | Meaning | Examples |
| --- | --- | --- |
| reset | Reset | Reset |
| rtoken | Restore to factory settings | Restore to factory settings |
| save | Save | Save settings |
| saver | Save & Reset |  |
| setcfg (x1) (x2) (x3) (x4) (x5) (x6)(x7) | Set up configuration | setcfg 1 1 1111 1 100 1 0 x1: Number of discovered tags (1) x2: Number of bound tags (1) x3: Personal network ID (0 - 0xfffe) x4: Base station ID (0 - 3) x5: Refresh rate (deprecated) x6: Filtering settings (deprecated) x7: Reporting format (0: General format / 1: Json format / 2: Car-following format / 3: Car positioning format, 4: General format (RSSI)) |
| getcfg | Get configuration |  |
| setslot(x1) | Set the time slot cycle (related to the refresh rate) | setslot 10 x1: A single time period is 10 milliseconds (Note: If one time slot is 10 milliseconds, the refresh rate is 100Hz) |
| getslot | Obtain the time slot period (related to the refresh rate) |  |
| setfilter(x1)(x2) | Set the hardware filtering parameters | setfilter 1 30 x1: Whether to enable hardware filtering x2: Hardware filtering coefficient (2 - 50) |
| getfilter | Obtain the hardware filtering parameters |  |
| addtag(x1)(x2)(x3)(x4)(x5) | Binding tag | addtag 10205FA01000154F 154F 0001 000A 00 x1: Long address of the tag (64 bits) x2: Short address of the tag x3: Fast refresh rate of the tag (deprecated) x4: Slow refresh rate of the tag (deprecated) x5: Tag mode (deprecated) |
| deltag | Delete the tag | deltag 10205FA01000154F x1: Label long address 64 bits |
| getklist | Obtain the list of already paired tags |  |
| getdlist | Obtain the list of tags that the request is to join. |  |